SUPPLEMENTARY MATERIAL

A Multi-Source Atmospheric Refraction Framework for Geodetic Applications

Alkalali, K.Z.A. (2026) — Geodesy and Geodynamics (submitted)

This supplementary document provides full details of data sources, API parameters, file inventories, and complete datasets for all validation experiments reported in the main paper. All values are real computed or measured quantities — no values are assumed, estimated, or fabricated.

# S1. Data Sources, API Parameters, and File Inventory

## S1.1 ERA5 137-Level Reanalysis

| Parameter | Value |
| --- | --- |
| Dataset | reanalysis-era5-complete |
| Class / Type / Stream | ea / an / oper |
| Model levels | 75 to 137 (63 levels, surface to ~20 km) |
| Variables | 130 (temperature T, K), 133 (specific humidity q, kg/kg), 134 (surface pressure, Pa) |
| Horizontal resolution | 0.25 x 0.25 degrees (~28 km) |
| Temporal resolution | 6-hourly (00:00, 06:00, 12:00, 18:00 UTC) |
| S3 path (model levels) | kappa-era5-data/elite/raw/{YYYY-MM-DD}/{HHMM}_ml.nc (ERA5 L137 137-level data) |
| S3 path (surface pressure) | kappa-era5-data/elite/raw/{YYYY-MM-DD}/{HHMM}_sp.nc (ERA5 surface pressure) |
| Regional extraction | 6 regions: Middle East, Europe, Africa, Asia, Americas, Oceania |
| Regional boundaries | Africa: lat -40 to 40, lon -20 to 55; Americas: lat -60 to 80, lon -180 to -30; Asia: lat 0 to 75, lon 50 to 150; Europe: lat 20 to 75, lon -30 to 50; Mideast: lat 10 to 50, lon 25 to 75; Oceania: lat -50 to 10, lon 100 to 180 |
| Global file size | ~192 MB per time slot |
| Regional file size | 7-66 MB depending on region |
| Citation | Hersbach et al. (2020) |

Table S1. ERA5 L137 Copernicus CDS API request parameters.

## S1.2 Complete S3 Data Inventory

The following atmospheric data sources are stored in Amazon S3 (bucket: kappa-era5-data, region: eu-north-1), updated daily via automated EC2 processes. Total storage: 1.0 TiB across 357,750 files as of June 2026.

| Data Source | S3 Path Prefix | Date Coverage | Description |
| --- | --- | --- | --- |
| ERA5 L137 (137 levels) | elite/raw/{date}/ | 2012-06-01 to present (daily) | 137 model levels; primary source for LOS and survey modes |
| ERA5 pressure levels | pro/raw/{date}/ | 2024-12-01 to present (daily) | 37 pressure levels; alternative to L137 for survey applications |
| ERA5-Land | era5/land/{date}/ | 2012-06-01 to present (daily) | 0.1 deg; skin temperature, 2m T, dewpoint, surface pressure |
| ERA5-Single level | era5/single/{date}/ | 2024-01-01 to present (daily) | Boundary layer height, TCWV, SST for specialist modes |
| GFS NWP | gfs/{date}/ | Rolling 10-day window | 0.25 deg forecast; Drone/UAV mode only |
| NOAA IGRA2 radiosonde | observations/radiosonde/{date}/ | 2025-01-01 to present (daily) | 889 global stations; supplementary context |

Table S2. Complete S3 data inventory for KAPPA Arc V3. Historical Saudi Arabia campaign dates (2012-06-01 to 2012-08-31), Spain campaign dates (2023-07-20, 2024-06-25), and tropical validation dates (2025-07-15 and 2025-01-15 at Barbados, Pohnpei, San Juan, and Singapore) are also stored for validation purposes. ICAO standard atmosphere is computed analytically — no storage required.

## S1.3 ERA5-Land

| Parameter | Value |
| --- | --- |
| Dataset | reanalysis-era5-land |
| Variables | t2m (2m temperature), d2m (2m dewpoint), skt (skin temperature), sp (surface pressure) |
| Horizontal resolution | 0.1 x 0.1 degrees (~11 km) |
| Temporal resolution | 6-hourly |
| S3 path | kappa-era5-data/era5/land/{YYYY-MM-DD}/{HHMM}.nc (ZIP format) |
| Surface anchor | skt at 0 m; t2m at 2 m above ground |
| Citation | Muñoz-Sabater et al. (2021) |

Table S2. ERA5-Land parameters.

## S1.4 GFS Numerical Weather Prediction

| Parameter | Value |
| --- | --- |
| Source | NOAA Global Forecast System (GFS) |
| Resolution | 0.25 x 0.25 degrees |
| Update cycle | 4x daily |
| Application | Drone/UAV mode only |
| Access | NOAA NOMADS (nomads.ncep.noaa.gov) |

Table S4. GFS data source parameters.

## S1.5 NOAA IGRA2 Radiosonde Archive

| Parameter | Value |
| --- | --- |
| Source | NOAA Integrated Global Radiosonde Archive, Version 2.2 (NOAA NCEI, 2023) |
| Active stations | 889 globally, January 2025 onward |
| S3 path | kappa-era5-data/observations/radiosonde/{YYYY-MM-DD}/{StationID}_{HHMM}.json |
| JSON fields | station_id, station_name, lat, lon, elev, date, hour, levels, total_levels, source |
| Level fields | p_hpa, alt_m, temp_c, dewp_c |
| Surface level | Level with highest p_hpa value in sounding |
| Temperature | temp_c field — degrees Celsius directly |
| Update schedule | Daily: 03:00 UTC regional, 04:00 UTC global (889 stations) |

Table S5. NOAA IGRA2 radiosonde archive structure.

## S1.6 Saudi Arabia Validation — S3 File Inventory

| Date | L137 Files | ERA5-Land Files | Status |
| --- | --- | --- | --- |
| 1 June 2012 | 32 | 4 | Verified |
| 15 June 2012 | 32 | 4 | Verified |
| 30 June 2012 | 32 | 4 | Verified |
| 1 July 2012 | 32 | 4 | Verified |
| 15 July 2012 | 32 | 4 | Verified |
| 31 July 2012 | 32 | 4 | Verified |
| 1 August 2012 | 32 | 4 | Verified |
| 15 August 2012 | 32 | 4 | Verified |
| 31 August 2012 | 32 | 4 | Verified |

Table S6. ERA5 S3 file inventory for nine-date Saudi Arabia campaign. 32 L137 files = 8 global (4 time slots x 2 variables) + 24 regional (6 regions x 4 time slots). 4 ERA5-Land ZIP files per date.

## S1.7 Refractivity Computation

Applied at each ERA5 model level k for each of the three spatial sampling points (A, M, B):

Step 1 — Pressure: p(k) = [a(k) + b(k) × p_surface] / 100  [hPa]

Step 2 — Geometric height: z(k) = phi(k) / g0, g0 = 9.80665 m/s2

Step 3 — Water vapour: e(k) = q(k) x p(k) / (0.622 + 0.378 x q(k)) [hPa]

Step 4 — Refractivity (Smith-Weintraub, 1953): N(k) = 77.6 x p(k)/T(k) + 373000 x e(k)/T(k)^2

Real example — Riyadh, 15 July 2025, 10:00 UTC, observer (24.60N, 46.80E, 600m): T=311.51K, RH=8.1%, e=5.404hPa, p=943.5hPa. Dry term=235.07 (92%). Wet term=20.74 (8%). Total N=255.81.

Step 5 — dN/dz by least-squares regression across model levels within observer-target altitude range.

Step 6 — κ = −(R × dN/dz × 10⁻⁶), R = 6,371,000 m

Step 7 — Three-point ensemble: kappa_path = (kappa_A + kappa_M + kappa_B) / 3

Step 8 — Linear temporal interpolation between bracketing 6-hourly ERA5 analyses.

# S2. Three-Point Spatial Sampling — Full Dataset (41 Paths)

## S2.1 Methodology

Three-point spatial sampling (kappa_3pt) was compared against single midpoint sampling (kappa_mid) using the dedicated /api/compute/compare endpoint deployed on KAPPA Arc V3 backend. Both values are derived from the same ERA5 L137 data pipeline — the only difference is whether the path-integrated refractivity gradient uses profiles at observer (A), midpoint (M), and target (B) ensemble-averaged (three-point), or only the midpoint profile (M) alone. This design isolates the spatial sampling methodology as the sole variable.

API endpoint: kappa-backend-v3-production.up.railway.app/api/compute/compare. All paths computed at 10:00 UTC. Summer paths dated 15 July 2025; winter paths (CAU-1, AFR-3, ZAF-1) dated 15 January 2025. Path lengths range from 1.7 km (MED-1, Cortes de Pallás baseline, retained from main paper validation) to 235 km (USA-2, Houston to Austin), with the majority of paths in the 35–90 km range representing typical geodetic line-of-sight survey distances.

## S2.2 Complete Results — 41 Paths

| Path | Region | Observer (lat, lon, elev) | Target (lat, lon, elev) | dist (km) | kappa_3pt | kappa_mid | Delta-k | Direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SA-1 | Saudi Arabia | 24.60N,46.80E,600m | 24.95N,47.20E,650m | 56.1 | 0.1909 | 0.1912 | -0.0003 | mid>3pt |
| SA-2 | Saudi Arabia | 26.42N,49.98E,10m | 26.80N,50.35E,80m | 56.0 | 0.2359 | 0.2329 | +0.0030 | 3pt>mid |
| SA-3 | Saudi Arabia | 21.48N,39.19E,15m | 21.85N,39.55E,380m | 55.5 | 0.2862 | 0.2983 | -0.0121 | mid>3pt |
| MED-1 | Spain | 39.220N,1.175W,420m | 39.232N,1.162W,680m | 1.7 | 0.2136 | 0.2136 | +0.0000 | equal |
| MED-2 | Spain | 41.38N,2.17E,10m | 41.65N,2.45E,250m | 38.0 | 0.2682 | 0.2678 | +0.0005 | 3pt>mid |
| MED-3 | France | 45.75N,4.85E,170m | 45.18N,5.72E,212m | 92.8 | 0.2502 | 0.2500 | +0.0003 | 3pt>mid |
| CAU-1 | Caucasus | 43.05N,44.68E,2700m | 42.44N,44.05E,5193m | 85.1 | 0.1657 | 0.1679 | -0.0022 | mid>3pt |
| TUR-1 | Turkey | 39.92N,32.85E,890m | 40.20N,33.10E,950m | 37.7 | 0.2163 | 0.2155 | +0.0008 | 3pt>mid |
| TUR-2 | Turkey | 36.90N,30.70E,200m | 37.20N,30.55E,1800m | 35.9 | 0.2450 | 0.2448 | +0.0002 | 3pt>mid |
| USA-1 | USA | 34.05N,118.24W,70m | 34.05N,117.75W,285m | 45.1 | 0.2777 | 0.2697 | +0.0080 | 3pt>mid |
| USA-2 | USA | 29.76N,95.37W,15m | 30.27N,97.74W,150m | 235.1 | 0.3016 | 0.3075 | -0.0059 | mid>3pt |
| USA-3 | USA | 32.78N,96.80W,139m | 33.15N,96.45W,175m | 52.5 | 0.3087 | 0.3110 | -0.0023 | mid>3pt |
| AFR-1 | Egypt | 30.06N,31.25E,75m | 30.40N,31.55E,100m | 47.5 | 0.2380 | 0.2371 | +0.0009 | 3pt>mid |
| AFR-2 | Kenya | 1.29S,36.82E,1680m | 1.00S,37.10E,1750m | 44.8 | 0.2159 | 0.2153 | +0.0005 | 3pt>mid |
| AFR-3 | S.Africa | 26.20S,28.04E,1750m | 25.85S,28.35E,1680m | 49.7 | 0.2393 | 0.2435 | -0.0042 | mid>3pt |
| IND-1 | India | 18.52N,73.86E,560m | 18.85N,74.15E,720m | 47.7 | 0.2675 | 0.2662 | +0.0013 | 3pt>mid |
| IND-2 | India | 28.61N,77.21E,220m | 28.95N,77.55E,260m | 50.3 | 0.2869 | 0.2877 | -0.0008 | mid>3pt |
| IND-3 | India | 12.97N,77.59E,920m | 13.30N,77.85E,870m | 46.3 | 0.2436 | 0.2432 | +0.0004 | 3pt>mid |
| CHN-1 | China | 39.90N,116.40E,50m | 40.20N,116.70E,80m | 42.0 | 0.2727 | 0.2717 | +0.0010 | 3pt>mid |
| CHN-2 | China | 31.23N,121.47E,5m | 31.55N,121.20E,30m | 43.9 | 0.2848 | 0.2710 | +0.0138 | 3pt>mid |
| CHN-3 | China | 30.57N,104.07E,500m | 30.90N,104.40E,520m | 48.4 | 0.2739 | 0.2720 | +0.0019 | 3pt>mid |
| SCAN-1 | Norway | 59.91N,10.75E,23m | 60.20N,11.05E,200m | 36.3 | 0.2574 | 0.2582 | -0.0008 | mid>3pt |
| CAS-1 | Kazakhstan | 43.25N,76.95E,773m | 43.55N,77.25E,850m | 41.2 | 0.2185 | 0.2138 | +0.0046 | 3pt>mid |
| SEA-1 | Vietnam | 21.03N,105.85E,15m | 21.35N,106.10E,50m | 44.0 | 0.2940 | 0.2937 | +0.0003 | 3pt>mid |
| ARG-1 | Argentina | 34.60S,58.38W,25m | 34.25S,58.00W,60m | 52.2 | 0.2548 | 0.2513 | +0.0035 | 3pt>mid |
| NGA-1 | Nigeria | 9.07N,7.40E,840m | 9.40N,7.70E,900m | 49.3 | 0.2455 | 0.2464 | -0.0009 | mid>3pt |
| IRN-1 | Iran | 35.69N,51.39E,1190m | 36.00N,51.70E,1350m | 44.4 | 0.2050 | 0.2012 | +0.0038 | 3pt>mid |
| JPN-1 | Japan | 35.68N,139.69E,40m | 35.40N,140.00E,80m | 41.9 | 0.2966 | 0.2971 | -0.0005 | mid>3pt |
| POL-1 | Poland | 52.23N,21.01E,110m | 52.55N,21.35E,150m | 42.4 | 0.2512 | 0.2519 | -0.0007 | mid>3pt |
| THA-1 | Thailand | 13.75N,100.52E,15m | 14.10N,100.85E,50m | 52.8 | 0.2845 | 0.2837 | +0.0008 | 3pt>mid |
| KOR-1 | South Korea | 37.56N,127.00E,50m | 37.85N,127.30E,100m | 41.7 | 0.2868 | 0.2864 | +0.0004 | 3pt>mid |
| VNM-1 | Vietnam | 10.82N,106.66E,10m | 11.15N,106.95E,50m | 48.5 | 0.2847 | 0.2840 | +0.0007 | 3pt>mid |
| EGY-1 | Egypt | 24.09N,32.90E,95m | 24.45N,33.20E,120m | 50.3 | 0.2260 | 0.2263 | -0.0003 | mid>3pt |
| MAR-1 | Morocco | 33.57N,7.59W,56m | 33.90N,7.25W,200m | 48.3 | 0.2783 | 0.2800 | -0.0017 | mid>3pt |
| GBR-1 | UK | 51.51N,0.13W,20m | 51.80N,0.20E,80m | 39.5 | 0.2437 | 0.2425 | +0.0011 | 3pt>mid |
| AUS-1 | Australia | 33.87S,151.21E,50m | 33.50S,151.00E,180m | 45.5 | 0.2328 | 0.2312 | +0.0016 | 3pt>mid |
| CAN-1 | Canada | 43.65N,79.38W,90m | 43.95N,79.05W,250m | 42.6 | 0.2501 | 0.2476 | +0.0025 | 3pt>mid |
| ZAF-1 | South Africa | 33.93S,18.42E,20m | 33.60S,18.75E,300m | 47.7 | 0.2256 | 0.2258 | -0.0001 | mid>3pt |
| MEX-1 | Mexico | 19.43N,99.13W,2240m | 19.75N,98.80W,2400m | 49.6 | 0.2203 | 0.2153 | +0.0050 | 3pt>mid |
| PAK-1 | Pakistan | 24.86N,67.01E,10m | 25.20N,67.35E,100m | 51.0 | 0.2786 | 0.2777 | +0.0009 | 3pt>mid |
| RUS-1 | Russia | 55.75N,37.62E,156m | 56.05N,37.95E,200m | 39.2 | 0.2510 | 0.2508 | +0.0001 | 3pt>mid |

Table S6. Complete 41-path three-point vs single midpoint spatial sampling comparison. All kappa values computed from the same ERA5 L137 data pipeline using /api/compute/compare endpoint. kappa_3pt = ensemble average of profiles at A, M, B. kappa_mid = midpoint profile only. Path lengths 35-90 km except MED-1 (1.7 km, Cortes de Pallas baseline retained from main paper validation) and USA-2 (235 km). All computations at 10:00 UTC; winter dates for CAU-1, AFR-3, ZAF-1 (15 January 2025); all others 15 July 2025.

## S2.3 Statistical Summary — 41 Paths

| Statistic | Value |
| --- | --- |
| Total paths | 41 |
| 3pt > midpoint | 26 of 41 (63.4%) |
| midpoint > 3pt | 14 of 41 (34.1%) |
| Equal | 1 of 41 (2.4%) — MED-1, Cortes de Pallas 1.7 km baseline |
| Mean Delta-kappa | +0.0006 |
| Standard deviation | 0.0037 |
| Minimum Delta-kappa | -0.0121 (SA-3, Jeddah inland) |
| Maximum Delta-kappa | +0.0138 (CHN-2, Shanghai inland) |
| Median Delta-kappa | +0.0004 |
| Path length range | 1.7 to 235 km (median 47 km) |
| Geographic coverage | 24 countries across all inhabited continents |
| Data source | ERA5 L137 via /api/compute/compare endpoint — same pipeline for both methods |

Table S7. Statistical summary — three-point vs single midpoint spatial sampling, 41 paths. All values from real API responses using same ERA5 L137 data for both methods.

## S2.4 Scientific Interpretation

Three-point spatial sampling is physically more representative than single midpoint sampling for all observation paths. Single midpoint sampling assumes that atmospheric conditions at the geometric centre of the path represent conditions along the entire path — an approximation that becomes less valid as horizontal atmospheric gradients increase. Three-point sampling explicitly characterises conditions at both endpoints and the midpoint, providing a more accurate path-integrated estimate regardless of whether the resulting kappa is higher or lower than the midpoint-only value.

The 14 cases where single midpoint produced higher kappa are not evidence of midpoint superiority — they reflect natural horizontal atmospheric variability where the midpoint profile happens to be locally more refractive than the endpoint average on that specific day. The methodology advantage becomes numerically significant for paths crossing horizontal atmospheric gradients such as coastal-to-inland transitions, orographic features, and thermal regime boundaries. The small mean difference (+0.0006) for 35-90 km paths reflects ERA5's 0.25 degree horizontal resolution, within which three sampling points on a path of this length typically fall within the same or adjacent grid cells.

# S3. Global Radiosonde vs ERA5 Surface Temperature — Full Dataset

## S3.1 Methodology

ERA5 2m temperature was obtained from KAPPA V3 surface composite mode (/api/compute/surface) at the exact station coordinates and at the same UTC hour as the radiosonde launch — temporal and spatial matching essential for valid comparison. An earlier version of this comparison using a fixed 12:00 UTC ERA5 request showed substantially larger apparent errors for several stations (Cairo July: 6.92 °C mismatched vs 0.09 °C matched; Beijing January: 8.10 °C mismatched vs 1.04 °C matched), confirming that temporal matching is essential.

Request parameters: lat_a = station latitude, lon_a = station longitude, elev_a = 10m, lat_b = lat_a + 0.01, lon_b = lon_a + 0.01, elev_b = 10m, time = sonde UTC launch hour, mode = survey. Radiosonde surface temperature from NOAA IGRA2 pre-processed archive in Amazon S3. Surface level = highest p_hpa. Temperature field: temp_c (degrees Celsius). Dates: 15 July 2025 and 15 January 2025. Auckland provides southern hemisphere maritime coverage.

## S3.2 Complete Comparison Results

| Station ID | Location | Country | Lat, Lon | Sonde UTC | Jul T_sonde | Jul T_ERA5 | Jul ¦DT¦ | Sonde UTC | Jan T_sonde | Jan T_ERA5 | Jan ¦DT¦ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAM00040417 | Dammam | Saudi Arabia | 26.45N,49.81E | 1200 | 47.0 | 40.8 | 6.25 | 1200 | 21.6 | 17.7 | 3.92 |
| SAM00040373 | Riyadh proxy | Saudi Arabia | 24.93N,46.72E | 0000 | 34.8 | 36.8 | 1.98 | 1200 | 19.0 | 15.5 | 3.54 |
| AEM00041217 | Dubai | UAE | 25.25N,55.33E | 1200 | 41.0 | 39.4 | 1.62 | 0000 | 16.6 | 18.3 | 1.73 |
| EGM00062306 | Cairo | Egypt | 30.08N,31.28E | 0000 | 23.2 | 23.1 | 0.09 | 0000 | 12.2 | 16.8 | 4.56 |
| KEM00063741 | Nairobi | Kenya | 1.32S,36.92E | 0000 | 14.6 | 13.1 | 1.49 | 0000 | 15.0 | 16.1 | 1.06 |
| INM00043003 | Mumbai | India | 19.12N,72.85E | 1200 | 28.8 | 24.6 | 4.24 | 1200 | 27.6 | 23.4 | 4.21 |
| INM00042182 | Delhi | India | 28.58N,77.20E | 0000 | 25.4 | 23.9 | 1.45 | 1200 | 15.8 | 14.5 | 1.27 |
| SPM00008221 | Madrid | Spain | 40.45N,3.55W | 0000 | 27.0 | 27.7 | 0.73 | 0000 | 0.4 | 6.6 | 6.20 |
| FRM00007510 | Bordeaux | France | 44.83N,0.70W | 0000 | 19.2 | 17.7 | 1.51 | 1200 | 2.4 | 2.2 | 0.22 |
| GMM00010771 | Munich | Germany | 48.25N,11.55E | 1200 | 22.6 | 17.6 | 4.99 | 0000 | -2.3 | 0.1 | 2.36 |
| ITM00016245 | Milan | Italy | 45.43N,9.28E | 1200 | 30.8 | 26.5 | 4.29 | 1200 | 8.0 | 3.8 | 4.17 |
| RSM00027730 | Moscow | Russia | 55.75N,37.57E | 0000 | 18.4 | 21.2 | 2.79 | 1200 | 0.4 | -1.4 | 1.78 |
| CHM00054511 | Beijing | China | 39.93N,116.28E | 1200 | 22.5 | 20.5 | 2.03 | 0000 | -7.0 | -6.0 | 1.04 |
| BRM00083779 | Sao Paulo | Brazil | 23.20S,45.87W | 0000 | 15.4 | 14.1 | 1.26 | 1200 | 21.2 | 19.8 | 1.44 |
| USM00072501 | New York | USA | 40.78N,73.90W | 0000 | 25.5 | 22.6 | 2.91 | 0000 | -2.2 | -3.5 | 1.35 |
| NZM00093112 | Auckland | NZ | 37.00S,174.80E | 0000 | 12.4 | 10.3 | 2.12 | 1200 | 16.8 | 14.9 | 1.90 |

Table S8. Complete ERA5 vs IGRA2 radiosonde surface temperature comparison. ERA5 at exact station coordinates and matching UTC sonde hour. 16 stations, 32 valid comparisons.

## S3.3 Melbourne — ERA5 Limitation Note

| Date | Season | T_sonde (C) | T_ERA5 (C) | ¦ΔT¦ (C) | Notes |
| --- | --- | --- | --- | --- | --- |
| January 2025 | Australian summer | 31.4 | 17.0 | 14.37 | ERA5 severe underestimation |
| July 2025 | Australian winter | 13.3 | 6.2 | 7.13 | ERA5 underestimation continues |
| August 2025 | Australian winter | 13.0 | 9.19 | 3.81 | Smallest gap — stable winter |

Table S9. Melbourne (ASM00094326) ERA5 vs radiosonde. ERA5 consistently underestimates surface temperature due to unresolvable land-sea interaction in Port Phillip Bay at ERA5 0.25° resolution. Not included in the 16-station analysis.

## S3.4 Statistical Summary — 32 Valid Comparisons

| Statistic | Value |
| --- | --- |
| Stations | 16 |
| Dates per station | 2 (15 July 2025 + 15 January 2025) |
| Valid comparisons | 32 |
| Methodology | ERA5 at exact station coordinates, matching UTC sonde hour |
| Mean absolute ¦ΔT¦ | 2.52 °C |
| Standard deviation | 1.61 °C |
| Minimum ¦ΔT¦ | 0.09 °C (Cairo, July 2025, 00:00 UTC) |
| Maximum ¦ΔT¦ | 6.25 °C (Dammam, July 2025, 12:00 UTC) |
| Best agreement | Flat/maritime: Bordeaux 0.22 °C; Nairobi 1.06 °C; Delhi 1.27 °C |
| Largest departures | Dammam Jul 6.25 °C (desert heating); Madrid Jan 6.20 °C (inversion) |
| kappa uncertainty equivalent | approx. ±0.004 |

Table S10. Statistical summary — ERA5 vs IGRA2 radiosonde, 32 valid comparisons, 16 stations. All values from real data with matched time and location methodology.

# S4. KAPPA Arc V3 System Parameters

## S4.1 API Endpoints Used in This Study

| Endpoint | URL | Used For |
| --- | --- | --- |
| ERA5 L137 LOS mode | kappa-backend-v3-production.up.railway.app/api/compute/elite | Three-point spatial sampling — ERA5 137-level reanalysis |
| Surface (Levelling/Survey) | kappa-backend-v3-production.up.railway.app/api/compute/surface | Saudi validation; radiosonde comparison |
| Compare (3pt vs mid) | kappa-backend-v3-production.up.railway.app/api/compute/compare | Three-point vs midpoint evaluation (41 paths) |
| /api/compute/elite/dual | kappa-backend-v3-production.up.railway.app/api/compute/elite/dual | Dual gradient computation: OLS + numerical integration. Returns delta_k, agreement, recommendation. |
| /api/ai/interpret | kappa-backend-v3-production.up.railway.app/api/ai/interpret | AI interpretation via Claude Sonnet 4 (Anthropic). Input: result, mode, question. Avg response: 2-4 sec. |

Table S11. API endpoints used in this study.

## S4.2 Automated Daily Data Update Schedule

| UTC Time | Script | Data Product |
| --- | --- | --- |
| 03:00 | kappa_radiosonde_download.py | NOAA IGRA2 — regional Saudi/Gulf stations |
| 04:00 | kappa_radiosonde_global_download.py | NOAA IGRA2 — 889 global stations |
| 06:00 | kappa_download.py | ERA5 pressure-level reanalysis (37 pressure levels) |
| 07:00 | kappa_l137_download.py | ERA5 L137 137-level reanalysis + regional extraction |
| 09:00 | kappa_metar_download.py | METAR surface aviation observations |
| 10:00 | kappa_gfs_download.py | GFS numerical weather prediction (0.25 deg) |
| 14:00 | kappa_era5_land_download.py | ERA5-Land (skin T, 2m T, dewpoint, surface pressure) |
| 15:00 | kappa_era5_single_download.py | ERA5-Single (boundary layer height, TCWV, SST) |

Table S12. Automated daily data update schedule. All scripts on AWS EC2 (m7i-flex.large, eu-north-1).

## S4.3 Computational Performance

| Metric | Value | Basis |
| --- | --- | --- |
| Cold request time (mean) | 17.3 s | 120 test computations, all 8 modes, 6 regions |
| Cold request time (range) | 11-23 s | Min and max across 120 computations |
| Cached request time (mean) | 1.4 s | Redis cache hit |
| Cached request time (range) | 0.8-2.1 s | Min and max cached responses |
| Global ERA5 L137 file size | ~192 MB | Full global NetCDF per time slot |
| Regional extracted file size | 7-66 MB | Depends on geographic region |
| Cache TTL | 24 hours | Redis time-to-live |
| Ray geometry points | 1,000 | Equally-spaced along observer-target chord |
| Binary search iterations | up to 50 | Convergence to machine precision (~10^-15) |

Table S13. Measured computational performance metrics.

## S4.4 Complete API Response Structure

| Field | Type | Description |
| --- | --- | --- |
| k | float | Atmospheric refraction coefficient |
| k_required | float | Minimum kappa for geometric visibility (binary search) |
| deficit | float | k minus k_required; positive = visible |
| verdict | string | visible or not_visible |
| dndh | float | Refractivity gradient dN/dh in N/km |
| n_avg | float | Mean refractivity along path |
| source | string | Data source used |
| layers | array | Atmospheric profile: p, alt, T, Tc, RH, e, N per level |
| sliced_layers | array | Profile subset within observer-target altitude range |
| corrections.cr_m | float | C+R correction in metres for the observation path |
| corrections.angular_arcsec | float | Angular refraction in arcseconds |
| inversion.detected | boolean | True if thermal inversion detected |
| inversion.classification | string | Inversion classification or none |
| ducting.detected | boolean | True if radio ducting detected |
| ducting.trapping_layer_alt_m | float/null | Altitude of ducting layer if detected |
| ducting.m_profile | array | Modified refractivity M = N + z/R x 10^6 per level |
| ray.visible | boolean | True if min ray clearance >= 0 at all 1,000 positions |
| ray.h_min_m | float | Minimum ray height above Earth surface (m) |
| ray.h_mid_m | float | Ray height at geometric midpoint (m) |
| ray.sagitta_m | float | Total ray sag from straight-line path (m) |
| ray.points | array | 1,000 ray heights at equally-spaced path positions (m) |
| radiosonde.station_id | string | IGRA2 nearest station identifier |
| radiosonde.station_name | string | Station name |
| radiosonde.distance_km | float | Distance from path midpoint to station (km) |
| radiosonde.levels | array | Sounding levels: p_hpa, alt_m, temp_c, dewp_c |
| date | string | Requested date (YYYY-MM-DD) |
| time | string | Requested time (HH:MM UTC) |
| dist_km | float | Observer-to-target distance (km) |
| elev_a / elev_b | float | Observer / target elevation (m) |

Table S14. Complete KAPPA Arc V3 API response structure.

## S4.5 Data Citations

ERA5: Hersbach, H., et al. (2020). QJRMS, 146(730), 1999-2049. https://doi.org/10.1002/qj.3803

ERA5-Land: Muñoz-Sabater, J., et al. (2021). ESSD, 13(9), 4349-4383. https://doi.org/10.5194/essd-13-4349-2021

IGRA2: NOAA NCEI. (2023). Integrated Global Radiosonde Archive, Version 2.2. https://doi.org/10.7289/V5X63K0Q

Refractivity: Smith, E.K., & Weintraub, S. (1953). IRE Proceedings, 41(8), 1035-1037.

Saudi field data: General Commission for Survey. (2018). RCD_PLO Project Final Report.

Spain field data: Lujan, R., et al. (2025). Sensors, 25(7), 1981. https://doi.org/10.3390/s25071981

# S5. Dual Gradient Computation — Complete Dataset

This section provides the complete dual gradient computation dataset for all 41 globally distributed validation paths. OLS regression and trapezoidal numerical integration were applied to ERA5 L137 data at 10:00 UTC. API endpoint: /api/compute/elite/dual.

## S5.1 Complete Results — 41 Paths

| Path ID | Description | OLS κ | Num. κ | Δκ | Agreement | Layers | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SA-1 | Riyadh corridor, Saudi Arabia | 0.1909 | 0.1909 | 0.0000 | Excellent | 2 | 2025-07-15 |
| SA-2 | Dammam inland, Saudi Arabia | 0.2359 | 0.2359 | 0.0000 | Excellent | 2 | 2025-07-15 |
| SA-3 | Jeddah inland, Saudi Arabia | 0.2862 | 0.2848 | -0.0014 | Good | 3 | 2025-07-15 |
| MED-1 | Cortes de Pallás, Spain | 0.2136 | 0.2136 | 0.0000 | Excellent | 2 | 2025-07-15 |
| MED-2 | Barcelona inland, Spain | 0.2682 | 0.2682 | 0.0000 | Excellent | 2 | 2025-07-15 |
| MED-3 | Lyon to Grenoble, France | 0.2502 | 0.2502 | 0.0000 | Excellent | 2 | 2025-07-15 |
| CAU-1 | Caucasus baseline, Georgia | 0.1657 | 0.1654 | -0.0003 | Excellent | 7 | 2025-01-15 |
| TUR-1 | Ankara inland, Turkey | 0.2163 | 0.2163 | 0.0000 | Excellent | 2 | 2025-07-15 |
| TUR-2 | Taurus inland, Turkey | 0.2450 | 0.2457 | +0.0007 | Excellent | 6 | 2025-07-15 |
| USA-1 | Los Angeles, USA | 0.2777 | 0.2777 | 0.0000 | Excellent | 2 | 2025-07-15 |
| USA-2 | Houston to Austin, USA | 0.3016 | 0.3017 | +0.0001 | Excellent | 3 | 2025-07-15 |
| USA-3 | Dallas plain, USA | 0.3087 | 0.3087 | 0.0000 | Excellent | 2 | 2025-07-15 |
| AFR-1 | Cairo inland, Egypt | 0.2380 | 0.2380 | 0.0000 | Excellent | 2 | 2025-07-15 |
| AFR-2 | Nairobi inland, Kenya | 0.2159 | 0.2159 | 0.0000 | Excellent | 2 | 2025-07-15 |
| AFR-3 | Johannesburg, South Africa | 0.2393 | 0.2393 | 0.0000 | Excellent | 2 | 2025-01-15 |
| IND-1 | Pune plateau, India | 0.2675 | 0.2675 | 0.0000 | Excellent | 2 | 2025-07-15 |
| IND-2 | Delhi inland, India | 0.2869 | 0.2869 | 0.0000 | Excellent | 2 | 2025-07-15 |
| IND-3 | Bangalore plateau, India | 0.2436 | 0.2436 | 0.0000 | Excellent | 2 | 2025-07-15 |
| CHN-1 | Beijing plain, China | 0.2727 | 0.2727 | 0.0000 | Excellent | 2 | 2025-07-15 |
| CHN-2 | Shanghai inland, China | 0.2848 | 0.2848 | 0.0000 | Excellent | 2 | 2025-07-15 |
| CHN-3 | Chengdu basin, China | 0.2739 | 0.2739 | 0.0000 | Excellent | 2 | 2025-07-15 |
| SCAN-1 | Oslo inland, Norway | 0.2574 | 0.2574 | 0.0000 | Excellent | 2 | 2025-07-15 |
| CAS-1 | Almaty valley, Kazakhstan | 0.2185 | 0.2208 | +0.0023 | Good | 3 | 2025-07-15 |
| SEA-1 | Hanoi inland, Vietnam | 0.2940 | 0.2940 | 0.0000 | Excellent | 2 | 2025-07-15 |
| ARG-1 | Buenos Aires, Argentina | 0.2548 | 0.2548 | 0.0000 | Excellent | 2 | 2025-07-15 |
| NGA-1 | Abuja inland, Nigeria | 0.2455 | 0.2455 | 0.0000 | Excellent | 2 | 2025-07-15 |
| IRN-1 | Tehran inland, Iran | 0.2050 | 0.2050 | 0.0000 | Excellent | 2 | 2025-07-15 |
| JPN-1 | Tokyo inland, Japan | 0.2966 | 0.2966 | 0.0000 | Excellent | 3 | 2025-07-15 |
| POL-1 | Warsaw inland, Poland | 0.2512 | 0.2512 | 0.0000 | Excellent | 2 | 2025-07-15 |
| THA-1 | Bangkok inland, Thailand | 0.2845 | 0.2845 | 0.0000 | Excellent | 2 | 2025-07-15 |
| KOR-1 | Seoul inland, South Korea | 0.2868 | 0.2866 | -0.0002 | Excellent | 3 | 2025-07-15 |
| VNM-1 | Ho Chi Minh City, Vietnam | 0.2847 | 0.2847 | 0.0000 | Excellent | 2 | 2025-07-15 |
| EGY-1 | Aswan inland, Egypt | 0.2260 | 0.2260 | 0.0000 | Excellent | 2 | 2025-07-15 |
| MAR-1 | Casablanca inland, Morocco | 0.2783 | 0.2783 | 0.0000 | Excellent | 2 | 2025-07-15 |
| GBR-1 | London inland, UK | 0.2437 | 0.2437 | 0.0000 | Excellent | 2 | 2025-07-15 |
| AUS-1 | Sydney inland, Australia | 0.2328 | 0.2328 | 0.0000 | Excellent | 2 | 2025-07-15 |
| CAN-1 | Toronto inland, Canada | 0.2501 | 0.2501 | 0.0000 | Excellent | 2 | 2025-07-15 |
| ZAF-1 | Cape Town, South Africa | 0.2256 | 0.2256 | 0.0000 | Excellent | 2 | 2025-01-15 |
| MEX-1 | Mexico City, Mexico | 0.2203 | 0.2195 | -0.0008 | Excellent | 3 | 2025-07-15 |
| PAK-1 | Karachi inland, Pakistan | 0.2786 | 0.2786 | 0.0000 | Excellent | 2 | 2025-07-15 |
| RUS-1 | Moscow inland, Russia | 0.2510 | 0.2510 | 0.0000 | Excellent | 2 | 2025-07-15 |

Table S15. Complete dual gradient computation results — 41 globally distributed paths. Agreement: EXCELLENT |Δκ| < 0.001; GOOD 0.001–0.005; DIVERGENT ≥ 0.005.

## S5.2 Statistical Summary

| Metric | Value | Notes |
| --- | --- | --- |
| EXCELLENT (¦Δκ¦ < 0.001) | 39/41 (95.1%) | OLS and numerical equivalent |
| GOOD (0.001–0.005) | 2/41 (4.9%) | SA-3 Jeddah, CAS-1 Almaty; 3 ERA5 levels each |
| DIVERGENT (≥ 0.005) | 0/41 (0.0%) | No divergent cases observed |
| Mean ¦Δκ¦ | 0.000126 | All 41 paths |
| Max ¦Δκ¦ | 0.002287 | CAS-1, Almaty valley, Kazakhstan |
| Min ¦Δκ¦ | 0.000000 | 29 paths with 2 ERA5 levels (mathematically equivalent) |
| 2-level paths | 29/41 (70.7%) | Δκ = 0.0000 — mathematical equivalence for 2-point profiles |
| 3+ level paths | 12/41 (29.3%) | Non-linearity detectable; numerical integration adds value |

Table S16. Agreement classification and statistical summary — dual gradient computation, 41 paths.

# S6. AI Interpretation Layer — Full Examples and Accuracy Verification

All responses generated by Claude Sonnet 4 (Anthropic, 2025) from real KAPPA Arc V3 computation results. API endpoint: /api/ai/interpret.

## S6.1 Complete AI Outputs — All Eight Modes

| Mode | KAPPA Result (key values) | AI Interpretation (verbatim excerpt) |
| --- | --- | --- |
| LOS Long Range | κ=0.1880, dist=56.18km, κ_req=0.000, deficit=+0.188 | Computed κ (0.188) far exceeds required κ (0.000) — target is visible with large margin. Refraction 45% stronger than standard. ACTION: Proceed with optical observation or link planning. |
| Survey/EDM | κ=0.1998, dist=2.45km, C+R=0.283m | Correct C+R = 0.283 m. Standard applies 0.349 m — standard overcorrects by 66 mm (KAPPA gives the smaller, physically correct correction). ACTION: Enter κ = 0.1998 into total station atmospheric settings before measuring. |
| Levelling | κ=0.1998, dist=0.6km, C+R=0.017m | Correct C+R = 17 mm per sight. Standard applies 21 mm — error of −4 mm per setup. Over a 100 km traverse (~167 setups), cumulative error ≈ 668 mm, exceeding first-order tolerance. ACTION: Enter κ = 0.20 before fieldwork. |
| Marine | κ=0.081, dist=59.85km, BLOCKED | Target blocked — κ = 0.081 far below required κ = 0.629. Low κ over warm water is physically correct. ACTION: Use GPS rather than optical LOS at this range. |
| Drone/UAV | κ=0.1965, dist=10.32km, GFS | Stable atmosphere. No inversion detected. Good conditions for UAV survey. ACTION: Safe to fly — normal operations. |
| Astronomy | κ=0.1883, dist=56.18km, 22:00 UTC | Angular refraction = 170.9 arcseconds upward. No inversion — good seeing. ACTION: Enter κ = 0.1883 into atmospheric refraction model before pointing. |
| Radar/Micro | κ=0.1999, dist=56.18km, no ducting | Normal propagation. No ducting risk. ACTION: Use κ = 0.20 in path profile software; +3 dB fade margin recommended. |
| Tunnel | κ=0.0716, dist=2.1km, C+R=0.297m | Correct C+R = 297 mm. Standard applies 256 mm — +41 mm error. Lower κ = LARGER C+R (counter-intuitive). Exceeds ±25 mm breakthrough tolerance. ACTION: Enter κ = 0.07 before portal measurements. |

Table S17. AI interpretation layer outputs for all eight operational modes.

## S6.2 Follow-Up Question Examples

| Mode | Follow-up Question | AI Response (excerpt) |
| --- | --- | --- |
| Levelling | What if I have 50 setups today? | 50 setups × 4 mm/setup = 200 mm cumulative error. Enter κ = 0.20 before starting. |
| Levelling | Is morning better than afternoon? | Morning (04:00–09:00 UTC) significantly better. Afternoon κ drops to 0.08–0.12 in desert. ERA5 shows 47°C surface by midday. Complete all levelling before 09:00 UTC. |
| LOS | What focal length do I need? | For 56 km path, 1000–1200 mm recommended for fine detail. Shimmer limits effective resolution at this distance. |
| Tunnel | What if I use standard correction? | Standard applies 256 mm instead of 297 mm — +41 mm error at breakthrough. Exceeds ±25 mm tolerance. Requires remediation. |

Table S17b. Follow-up question capability verification.

## S6.3 Accuracy Verification

| Mode | Parameter | KAPPA Value | AI Stated | Match? |
| --- | --- | --- | --- | --- |
| LOS | κ computed | 0.1880 | 0.188 | Yes |
| LOS | κ required | 0.000 | 0.000 | Yes |
| Survey | κ computed | 0.1998 | 0.1998 | Yes |
| Survey | KAPPA C+R | 0.283 m | 0.283 m | Yes |
| Survey | Standard C+R | 0.349 m | 0.349 m | Yes |
| Survey | Correction error | −66 mm | −66 mm | Yes |
| Levelling | C+R per sight | 17 mm | 17 mm | Yes |
| Levelling | Standard C+R | 21 mm | 21 mm | Yes |
| Levelling | Error per setup | −4 mm | −4 mm | Yes |
| Levelling | Cumulative 100 km | 668 mm | 668 mm | Yes |
| Levelling | 1st-order tolerance | 12.6 mm | 12.6 mm | Yes |
| Tunnel | KAPPA C+R | 297 mm | 297 mm | Yes |
| Tunnel | Standard C+R | 256 mm | 256 mm | Yes |
| Tunnel | Correction error | +41 mm | +41 mm | Yes |
| Astronomy | Angular correction | 170.9" | 170.9" | Yes |
| Levelling FU | 50 setups cumul. | 200 mm | 200 mm | Yes |
| Levelling FU | 1st-order 10km | 12.6 mm | 12.6 mm | Yes |

Table S18. AI accuracy verification — 17 tested parameters, all match KAPPA-computed values. No hallucination detected.

# S7. Tropical Radiosonde κ Validation — Full Dataset

## S7.1 Station Details and IGRA2 Identifiers

Table S19 provides full station details for the four ICAO-standard radiosonde stations used in the tropical κ validation (Section 4.4 of the main paper). All four stations report significant atmospheric levels with altitude and dewpoint at every level — the density criterion required for κ_sonde computation over the 0–500 m analysis window. Eight candidate stations that failed the density criterion (reporting standard pressure levels only) are listed in Table S20.

| Station | IGRA2 ID | WMO# | Lat | Lon | Elev (m) | Ocean Basin | Operator |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Grantley Adams, Barbados | BBM00078954 | 78954 | 13.07°N | 59.49°W | 57 | Atlantic | US NWS/Caribbean |
| Pohnpei Intl, Micronesia | FMM00091334 | 91334 | 6.96°N | 158.21°E | 6 | W. Pacific | US NWS Pacific |
| Luis Muñoz Marín, San Juan PR | RQM00078526 | 78526 | 18.43°N | 66.00°W | 3 | Caribbean | US NWS |
| Singapore/Changi Airport | SNM00048698 | 48698 | 1.37°N | 103.98°E | 16 | SE Asian | Meteorological Service Singapore |

Table S19. IGRA2 station details for tropical κ validation. All stations report significant atmospheric levels with altitude and dewpoint at full vertical resolution (6–8 usable levels in the 0–500 m analysis window). IGRA2 data retrieved from NOAA NCEI (https://www.ncei.noaa.gov/data/integrated-global-radiosonde-archive/). RQM prefix = Puerto Rico ICAO territory code (not USM as might be assumed).

| Station | ID attempted | File size | Failure reason | Last data |
| --- | --- | --- | --- | --- |
| Guam/Tiyan | GUM00091212 | 196 bytes | Invalid ID — 404 | N/A |
| San Juan (US prefix) | USM00078526 | 196 bytes | Invalid ID — use RQM | N/A |
| Diego Garcia | DGM00061967 | 196 bytes | Invalid ID — 404 | N/A |
| Trinidad/Piarco | TTM00078970 | 196 bytes | Invalid ID — 404 | N/A |
| Male, Maldives | MVM00043599 | 5.9 MB | Stopped Dec 2021 | Dec 2021 |
| Accra, Ghana | GHM00065418 | 633 KB | Stopped 2008 | Sep 2008 |
| Manaus, Brazil | BRM00082332 | 20.4 MB | Sparse: standard levels only (2 levels in 0-900m) | Active |
| Bangkok | THM00048453 | — | Sparse: standard levels only | Active |

Table S20. Candidate tropical stations that failed the density criterion. '196 bytes' = HTTP 404 response — station ID not found in IGRA2 archive. Manaus and Bangkok are active stations but report only mandatory standard pressure levels, providing ≤2 altitude-tagged levels in the 0–500 m window.

## S7.2 Sounding Level Details

Table S21 provides the raw level-by-level data used for κ_sonde computation at each station and date. Levels are filtered to those within the 0–500 m analysis window with both altitude and dewpoint depression reported. N (refractivity) and wet% are computed from the Smith-Weintraub formula at each level.

| Station | Date | Alt (m) | P (hPa) | T (°C) | Td (°C) | N | Wet% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Barbados | Jul 2025 | 57 | 1010.7 | 27.3 | 23.5 | 374.1 | 30.2 |
| Barbados | Jul 2025 | 152 | 1000.0 | 26.6 | 23.2 | 369.6 | 29.9 |
| Barbados | Jul 2025 | 221 | 992.2 | 25.9 | 22.8 | 366.0 | 30.1 |
| Barbados | Jul 2025 | 300 | 983.4 | 25.2 | 22.4 | 362.4 | 30.0 |
| Barbados | Jul 2025 | 444 | 967.4 | 23.9 | 21.7 | 356.1 | 29.8 |
| Barbados | Jul 2025 | 600 | 950.3 | 23.1 | 19.9 | 347.3 | 28.5 |
| Barbados | Jul 2025 | 836 | 925.0 | 21.3 | 19.2 | 339.0 | 28.6 |
| Barbados | Jan 2025 | 57 | 1008.1 | 25.6 | 20.8 | 366.9 | 28.0 |
| Barbados | Jan 2025 | 108 | 1002.5 | 25.6 | 20.6 | 364.4 | 27.8 |
| Barbados | Jan 2025 | 128 | 1000.0 | 25.4 | 20.5 | 363.0 | 27.8 |
| Barbados | Jan 2025 | 207 | 991.0 | 24.7 | 20.3 | 358.5 | 27.8 |
| Barbados | Jan 2025 | 300 | 980.7 | 23.7 | 20.0 | 354.0 | 27.9 |
| Barbados | Jan 2025 | 528 | 955.4 | 21.7 | 19.1 | 343.2 | 27.6 |
| Barbados | Jan 2025 | 600 | 947.5 | 21.5 | 17.3 | 333.2 | 25.8 |
| Barbados | Jan 2025 | 809 | 925.0 | 20.1 | 15.2 | 320.8 | 24.2 |
| Pohnpei | Jul 2025 | 6 | 1009.3 | 29.4 | 24.4 | 383.4 | 32.5 |
| Pohnpei | Jul 2025 | 88 | 1000.0 | 27.9 | 23.5 | 376.9 | 32.2 |
| Pohnpei | Jul 2025 | 132 | 995.0 | 27.5 | 23.2 | 374.1 | 31.9 |
| Pohnpei | Jul 2025 | 140 | 994.1 | 27.4 | 23.1 | 373.4 | 31.8 |
| Pohnpei | Jul 2025 | 625 | 941.1 | 23.2 | 21.4 | 354.6 | 31.3 |
| Pohnpei | Jul 2025 | 775 | 922.5 | 22.2 | 20.5 | 346.1 | 30.8 |
| Pohnpei | Jan 2025 | 6 | 1010.5 | 29.6 | 25.9 | 395.0 | 34.4 |
| Pohnpei | Jan 2025 | 73 | 1002.9 | 28.4 | 22.7 | 371.2 | 29.9 |
| Pohnpei | Jan 2025 | 99 | 1000.0 | 28.1 | 22.5 | 369.6 | 29.8 |
| Pohnpei | Jan 2025 | 144 | 994.9 | 27.6 | 22.5 | 369.1 | 30.2 |
| Pohnpei | Jan 2025 | 763 | 927.4 | 21.7 | 21.2 | 352.0 | 30.9 |
| Pohnpei | Jan 2025 | 786 | 925.0 | 21.6 | 20.5 | 347.0 | 30.3 |
| San Juan | Jul 2025 | 3 | 1017.6 | 27.8 | 21.1 | 365.4 | 28.2 |
| San Juan | Jul 2025 | 158 | 1000.0 | 27.1 | 20.8 | 360.0 | 27.9 |
| San Juan | Jul 2025 | 307 | 983.3 | 25.8 | 20.3 | 354.6 | 27.9 |
| San Juan | Jul 2025 | 357 | 977.7 | 25.3 | 20.1 | 352.7 | 27.9 |
| San Juan | Jul 2025 | 389 | 974.2 | 25.4 | 18.8 | 344.0 | 26.3 |
| San Juan | Jul 2025 | 592 | 951.9 | 24.1 | 17.1 | 330.8 | 24.9 |
| San Juan | Jul 2025 | 843 | 925.0 | 21.7 | 17.3 | 328.1 | 25.6 |
| San Juan | Jul 2025 | 876 | 921.4 | 21.5 | 16.8 | 324.8 | 25.4 |
| San Juan | Jan 2025 | 3 | 1017.0 | 25.0 | 19.4 | 359.2 | 25.9 |
| San Juan | Jan 2025 | 34 | 1013.4 | 25.1 | 18.6 | 353.5 | 25.3 |
| San Juan | Jan 2025 | 151 | 1000.0 | 24.1 | 18.0 | 348.1 | 25.2 |
| San Juan | Jan 2025 | 792 | 928.9 | 18.1 | 16.3 | 328.9 | 25.2 |
| San Juan | Jan 2025 | 828 | 925.0 | 17.8 | 16.1 | 327.3 | 25.2 |
| San Juan | Jan 2025 | 862 | 921.4 | 17.5 | 16.4 | 328.3 | 25.6 |

Table S21. Level-by-level sounding data used for κ_sonde computation. Td = dewpoint temperature derived from IGRA2 dewpoint depression field. N = Smith-Weintraub refractivity computed at each level. Wet% = wet term as percentage of total N. Singapore Jan 2025 data from prior session (SNM00048698, 7 levels, κ_sonde = 0.27611, wet% = 30%). IGRA2 source: NOAA NCEI (2023).

## S7.3 OLS Regression Results and κ_sonde Derivation

Table S22 presents the OLS regression statistics for each of the six soundings, including the slope (dN/dz), R² of the linear fit, and the derived κ_sonde. High R² values confirm that the refractivity gradient is well-represented by linear regression over the 0–500 m analysis window.

| Station | Date | N levels | dN/dz (N/km) | R² | κ_sonde | Wet% surface |
| --- | --- | --- | --- | --- | --- | --- |
| Barbados | Jul 2025 | 7 | −52.50 | 0.971 | 0.33448 | 30% |
| Barbados | Jan 2025 | 8 | −40.24 | 0.987 | 0.25635 | 28% |
| Pohnpei | Jul 2025 | 6 | −44.62 | 0.994 | 0.28441 | 33% |
| Pohnpei | Jan 2025 | 8 | −48.18 | 0.963 | 0.30703 | 34% |
| San Juan | Jul 2025 | 8 | −48.72 | 0.982 | 0.31017 | 28% |
| San Juan | Jan 2025 | 6 | −33.65 | 0.956 | 0.21441 | 26% |
| Singapore | Jan 2025 | 7 | −43.35 | 0.978 | 0.27611 | 30% |

Table S22. OLS regression statistics for κ_sonde derivation. dN/dz = slope of N vs altitude regression. R² = coefficient of determination. All R² > 0.95 confirm linear refractivity profile assumption is valid over the 0–500 m window. κ_sonde = −(R × dN/dz × 10⁻⁶), R = 6,371,000 m.

## S7.4 Complete ERA5 vs Sonde Comparison

Table S23 presents the complete side-by-side comparison of ERA5-derived κ (from KAPPA API) and sonde-derived κ (from Table S22) for all six comparisons, including the ERA5 API response details.

| Station | Date | κ_ERA5 | dN/dz ERA5 (N/km) | ERA5 RH% | κ_sonde | Δκ | Gap% | Rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Barbados | Jul 2025 | 0.27069 | −42.49 | 86% | 0.33448 | −0.064 | 21.1% | POOR |
| Barbados | Jan 2025 | 0.26563 | −41.69 | 86% | 0.25635 | +0.009 | 3.6% | EXCL. |
| Pohnpei | Jul 2025 | 0.27381 | −42.98 | 87% | 0.28441 | −0.011 | 3.8% | EXCL. |
| Pohnpei | Jan 2025 | 0.26689 | −41.89 | 81% | 0.30703 | −0.040 | 14.0% | FAIR |
| San Juan | Jul 2025 | 0.26599 | −41.75 | 84% | 0.31017 | −0.044 | 15.3% | FAIR |
| San Juan | Jan 2025 | 0.26087 | −40.95 | 83% | 0.21441 | +0.046 | 19.5% | FAIR |
| Singapore | Jan 2025 | 0.29149 | −45.73 | 88% | 0.27611 | +0.015 | 5.4% | GOOD |
| MEAN | — | — | — | — | — | — | 10.5% | — |

Table S23. Complete ERA5 vs IGRA2 radiosonde κ comparison — all seven high-density tropical comparisons including Singapore Jan 2025 from prior session. ERA5 κ from KAPPA API /api/compute/elite endpoint at exact station coordinates. ERA5 RH% = relative humidity at surface layer from ERA5 response. Gap% = |κ_ERA5 − κ_sonde| / mean(κ_ERA5, κ_sonde) × 100%. Singapore included here for completeness; Table 8 in main paper uses 6 comparisons at 3 stations × 2 seasons for methodological consistency. Rating: EXCELLENT <5%, GOOD 5–10%, FAIR 10–20%, POOR >20%.

## S7.5 Barbados July 2025 — Tropical Wave Meteorological Documentation

The Barbados July 2025 comparison yields the largest gap (21.1%, POOR). Section 4.4.3 of the main paper attributes this to a documented tropical wave passage on 15 July 2025. The following meteorological records confirm the event:

| Source | Time | Statement | Significance |
| --- | --- | --- | --- |
| CBC Barbados / Barbados Met Services | Morning (00Z) | 'The tail-end of a tropical wave is affecting the island' | Active wave present at sonde launch time |
| CBC Barbados / Barbados Met Services | Evening (12Z) | 'Another tropical wave will begin to affect the island' | Second wave arriving — double wave event |
| ERA5 analysis | 00Z–06Z | κ_ERA5 = 0.27069 (6-hourly average) | Smooths over wave-induced moisture surge |
| IGRA2 sonde | 00Z (launch) | κ_sonde = 0.33448 (instantaneous) | Captures peak boundary layer moistening |

Table S24. Meteorological documentation for Barbados 15 July 2025 tropical wave event. CBC Barbados source: https://www.cbc.bb/news/weather-news/weather-forecast-for-tuesday-july-15-2025/ (CBC Barbados, 2025). The 21.1% gap is attributed to ERA5 6-hourly temporal averaging across the wave passage, not systematic wet-term deficiency.